Jim Rosemary (rosemary@eecis.udel.edu)

This code was taken from a terminal emulator that I wrote a while

back. The entire program was written in machine language and each

revision was burned into an EPROM for the target system. Because of

this, I did not try to make the code too 'pretty' - I just wanted to

get it running. Read at your own risk.

Also, be advised that not all keyboards work with my machine. The better

of the two keyboards that I have tried works fine, whereas the cheap one

will not work at all.

It is advisable to obtain the keyboard docs from ftp.ee.ualberta.ca

in the /pub/cookbook directory and read them.

A 6522 is mapped at $fe30

PB0 goes to DATA

PB1 goes to CLOCK

Also, one of the control lines (CB1 or CB2, whichever corresponds

to Interrupt enable $10)

No interrupt code is provided here; only the actual transfer.

$ff is used for passing data to/from the Get Byte and Write Byte routines.

Reset has no parameters.

Get Byte:

lda $fe32

and #fc

sta $fe32

jsr get_bit

ldy #$09

loop:		jsr get_bit

ror $ff

dey

bne loop

jsr get_bit

lda $ff

rts

get_bit:

lda #$02

wait_for_hi	bit $fe30

bne wait_for_hi

lda $fe30

pha

lda #$02

wait_for_low	bit $fe30

bne wait_for_low

pla

lsr

rts

Write Byte:

lda $fe30

and #$fc

sta $fe30

lda $fe32

ora #03

sta $fe32

ldx #$c0

wait:		dex

bne wait

lda $fe32		// Write start bit

and #$fd

sta $fe32

lda #$02

wait_for_hi	bit $fe30		// Wait for clock line

beq wait_for_hi

wait_for_lo	bit $fe30

bne wait_for_lo

ldx #$08

lda $fe32

ldy #$01

loop		lsr $ff

jsr write_bit		// Write data bits

dex

bne loop

tya

lsr			// Write parity

jsr write_bit

sec			// Write stop bit

jsr write_bit

lda #$01

wait_lo		bit $fe30		// Wait for handshake from kbd

bne wait_lo

rts

write_bit

lda $fe32

and #$fe

bcs write_one

ora #$01

iny

write_one	sta $fe32

lda #$02

wait_for_lo	bit $fe30

bne wait_for_lo

wait_for_hi	bit $fe30

beq wait_for_hi

rts

Reset Keyboard:

lda $fe30

and #$fc

sta $fe30

lda $fe32

ora #$03

sta $fe32

ldx #$c0

wait		dex

bne wait

lda $fe32

and #$fd

sta $fe32

lda #$02

wait_for_hi	bit $fe30

beq wait_for_hi

wait_for_lo	bit $fe30

bne wait_for_lo

ldx #$08

lda $fe32

and #$fe

sta $fe32

lda #$02

wait_for_hi1	bit $fe30

beq wait_for_hi1

wait_for_lo1	bit $fe30

bne wait_for_lo1

dex

bne wait_for_hi1

lda #$02

wait_for_hi2	bit $fe30

beq wait_for_hi2

wait_for_lo2	bit $fe30

beq wait_for_lo2

lda #$01

wait_for_lo3	bit $fe30

bne wait_for_lo3

lda #$10

sta $fe3d

rts